| Iotronics Techlab Pvt Ltd. |  |
| --- | --- |
| Aim: | To design and implement a door lock system |
| Requirements: | Automation Kit, Mechatronics kit, Wireless & iot kit and Power and Control |
| IDE: | Arduino IDE |
| Connection Diagram: |  |
| Working | - The user inputs a password through the 4x4 keypad. - The Arduino Uno reads the input from the keypad. - If the entered password matches the predefined one, it triggers the servo motor to unlock the door. - If the password is incorrect, nothing happens. - The system resets after a predefined timeout period. |
| Procedure: | 1.Connect the Servo Motor: - Connect the control wire of the servo motor to a PWM pin on the Arduino Uno (e.g., pin 9). - Connect the power and ground wires of the servo motor to the 5V and GND pins on the Arduino Uno, respectively. 2.Connect the 4x4 Keypad: - Connect the rows and columns of the keypad to digital pins on the Arduino Uno. - Refer to the datasheet of the keypad for pin mappings. 3.Writing the Code: - Open the Arduino IDE. - Write code to read input from the 4x4 keypad. - Define a password for the door lock system. - Compare the entered password with the predefined one. - If the password matches, trigger the servo motor to unlock the door. 4.Uploading and Testing: - Upload the code to the Arduino Uno. - Test the system by entering the correct and incorrect passwords. - Ensure that the servo motor behaves as expected based on the input. |
| CODE: | #include <Keypad.h> #include <Wire.h> #include <LiquidCrystal_I2C.h> #include <Servo.h> Servo myservo; LiquidCrystal_I2C lcd(0x27, 16, 2); // Change the address if necessary #define Password_Lenght 7 // Give enough room for six chars + NULL char int pos = 0; // variable to store the servo position char Data[Password_Lenght]; // 6 is the number of chars it can hold + the null char = 7 char Master[Password_Lenght] = "123456"; //Change PASSWORD here byte data_count = 0, master_count = 0; bool Pass_is_good; char customKey; const byte ROWS = 4; const byte COLS = 3; char keys[ROWS][COLS] = { {'1', '2', '3'}, {'4', '5', '6'}, {'7', '8', '9'}, {'*', '0', '#'} }; bool door = true; byte rowPins[ROWS] = {64, 62, 68, 66}; //connect to the row pinouts of the keypad byte colPins[COLS] = {65, 63, 69, }; //connect to the column pinouts of the keypad Keypad customKeypad( makeKeymap(keys), rowPins, colPins, ROWS, COLS); //initialize an instance of class NewKeypad void setup() { Wire.begin(); lcd.init(); lcd.backlight(); myservo.attach(14); ServoClose(); } void loop() { if (door == 0) { customKey = customKeypad.getKey(); if (customKey == '#') { lcd.clear(); lcd.setCursor (3,0); lcd.print("LOCKING"); delay (700); lcd.setCursor (10,0); lcd.print("."); delay (800); lcd.setCursor (11,0); lcd.print("."); delay (800); lcd.setCursor (12,0); lcd.print("."); delay (500); lcd.clear(); delay (500); ServoClose(); lcd.setCursor (5,0); lcd.print("LOCKED"); delay (1700); door = 1; } } else Open(); } void clearData() { while (data_count != 0) { // This can be used for any array size, Data[data_count--] = 0; //clear array for new data } return; } void ServoOpen() { for (pos = 180; pos >= 0; pos -= 5) { // goes from 0 degrees to 180 degrees // in steps of 1 degree myservo.write(pos); // tell servo to go to position in variable 'pos' delay(15); // waits 15ms for the servo to reach the position } } void ServoClose() { for (pos = 0; pos <= 180; pos += 5) { // goes from 180 degrees to 0 degrees myservo.write(pos); // tell servo to go to position in variable 'pos' delay(15); // waits 15ms for the servo to reach the position } } void Open() { lcd.setCursor(1, 0); lcd.print("INPUT PASSWORD"); customKey = customKeypad.getKey(); if (customKey) // makes sure a key is actually pressed, equal to (customKey != NO_KEY) { lcd.setCursor(0, 1); lcd.print("******"); //To hide your PASSWORD, make sure its the same lenght as your password Data[data_count] = customKey; // store char into data array lcd.setCursor(data_count, 1); // move cursor to show each new char lcd.print(Data[data_count]); // print char at said cursor data_count++; // increment data array by 1 to store new char, also keep track of the number of chars entered } if (data_count == Password_Lenght - 1) // if the array index is equal to the number of expected chars, compare data to master { if (!strcmp(Data, Master)) // equal to (strcmp(Data, Master) == 0) { lcd.clear(); ServoOpen(); lcd.print(" ACCESS GRANTED"); lcd.setCursor(0,1); lcd.print("press # to close"); door = 0; } else { lcd.clear(); lcd.setCursor(1,0); lcd.print("ACCESS DENIED!"); delay(1500); door = 1; } clearData(); } } |
| Result/Output |  |