| Iotronics Techlab Pvt Ltd. |  |
| --- | --- |
| Aim: | To design and implement a basic calculator using Arduino, a 4x4 keypad, and an LCD display. |
| Requirements: | Automation kit (Lcd), Wireless and iot (4x4Key pad ) |
| IDE: | Arduino IDE |
| Connection Diagram: |  |
| Working | - Keypad allows input of numbers and arithmetic operators. - Arduino processes keypad input and performs calculations. - Results are displayed on the LCD screen. - Basic arithmetic operations like addition, subtraction, multiplication, and division are supported. |
| Procedure: | - Setting Up the Hardware: - Connect the 4x4 keypad to the Arduino Uno using jumper wires. - Connect the LCD display to the Arduino Uno using jumper wires. - Power the components using the appropriate power source. - Programming the Arduino Uno: - Open the Arduino IDE on your computer. - Write the code to read input from the keypad and display output on the LCD. - Implement functions to perform basic arithmetic operations based on keypad input. - Testing the Hardware: - Upload the code to the Arduino Uno. - Test each button on the keypad to ensure proper input recognition. - Verify that the LCD display shows the correct output for arithmetic operations. |
| CODE: | #include <Key.h> #include <Keypad.h> #include <Wire.h> #include <hd44780.h> // main hd44780 header #include <hd44780ioClass/hd44780_I2Cexp.h> // i2c expander i/o class header //LCD I2C Initialization hd44780_I2Cexp lcd; // declare lcd object: auto locate & config exapander chip // LCD geometry const int LCD_COLS = 16; const int LCD_ROWS = 2; double num1,num2 ; double total; char operation,button; const byte ROWS = 4; // Four rows const byte COLS = 4; // Three columns // Define the Keymap char keys[ROWS][COLS] = { {'1','2','3','+'}, {'4','5','6','-'}, {'7','8','9','*'}, {'=','0','%','/'} }; byte rowPins[ROWS] = { 64, 62, 68, 66 };// Connect keypad ROW0, ROW1, ROW2 and ROW3 to these Arduino pins. byte colPins[COLS] = { 65, 63, 69, 67 }; // Connect keypad COL0, COL1 and COL2 to these Arduino pins. Keypad kpd = Keypad( makeKeymap(keys), rowPins, colPins, ROWS, COLS ); // Create the Keypad void domath() // Simple switch case to pick what operation to do, based on button pressed by user. { switch(operation) { case '+': // Addition total = num1+num2; break; case '-': // Subtraction total = num1-num2; break; case '/': // Division. total = num1/num2; break; case '*': // Multiplication total = num1*num2; break; case '%': // Modulus total = (int)num1%(int)num2; break; } // Based on case selected print our total and lcd.setCursor(0,1); lcd.print('='); lcd.setCursor(1,1); lcd.print(total); } void setup() { int status; // initialize LCD with number of columns and rows: // hd44780 returns a status from begin() that can be used to determine if initalization failed. status = lcd.begin(LCD_COLS, LCD_ROWS); if(status) // non zero status means it was unsuccesful { status = -status; // convert negative status value to positive number // begin() failed so blink error code using the onboard LED if possible hd44780::fatalError(status); // does not return } // initalization was successful, the backlight should be on now lcd.setCursor(0, 0); lcd.print("Welcome"); lcd.setCursor(0, 1); lcd.print("Iotronics Pvt.Ltd"); delay(3000); lcd.clear(); // Print a message to the LCD lcd.print(" Calculator "); lcd.setCursor(0, 1); lcd.print(" Enter Number "); } void loop() { // Loops are convenient for reading key press from keypad while(1) // First loop. Composing first number. Until operator pressed { button = kpd.getKey(); // Button read if (button >='0' && button <='9') // If user pressed numeric value, 1 character at a time. { lcd.clear(); num1 = num1*10 + (button -'0'); //First operand composing lcd.setCursor(0,0); // Select first row on lcd lcd.print(num1); // Print current number1 } //Inputting operator if (num1 !=0 && (button=='+' ¦¦ button=='-' ¦¦ button=='*' ¦¦ button=='/' ¦¦ button=='%')) { operation = button; // operation remembers what mathematical operation user wants on numbers lcd.setCursor(0,1); // set cursor to row 2 lcd.print(operation); // print our operator break; } } while(1) // Second loop. Inputting second operand. Until '=' pressed { button = kpd.getKey(); // Button read if (button >='0' && button <='9') // Getting chars from keypad for second number { num2 = num2*10 + (button -'0'); //Second operand composing lcd.setCursor(1,1); // Select second row, second column on lcd lcd.print(num2); // Print current operand2 } if (button == '=' && num2 !=0) //Do calculation if '=' is pressed { domath(); //Calls domath() subroutine break; } } while(1) // Clearing screen for next calculation { //Waits ‘=’ key to be pressed so it can reset program and start over. button = kpd.getKey(); if (button =='=') { lcd.clear(); lcd.setCursor(0,0); lcd.print(" ENTER NUMBER "); num1=0; num2=0; total=0; operation=0; break; } } } |
| Result/Output |  |